[image: image1.jpg]



Product Advantages:                                                                                                                                                     

1. Lower fuel consumption and instant heating.

2. Compact structure is convenient to install.

3. No noise operation to ensure comfortable driving.

4. Continuous functional monitoring to reduce diagnosing time.

Application Scope: Various vehicles with gasoline and diesel as the fuel.

Product Specifications
Product model:        MC5.0-H/Q1  H/Q2     

Output power:             5.0KW

Fuel consumption:          0.42L/h

Voltage:                  12V

Temperature range:         -40℃— 80℃
Weight:                   2.12KG

Specification: 222(length)*90(wide)*174(high)mm

Heating mode: Transmitting the cooling liquid by energy to heat the engine      

Scope of application: 90#、93#、97# Petrol、0#、10#、-35# Diesel
Maintenance
	Fault Code(ER)
	Implication
	Solution

	ER01
	Power Breakdown
	Low battery , check the battery

	ER02
	Motor Breakdown
	Motor brake/Motor short circuit or Motor Ampere overlarge

	ER03
	Fuel pump Breakdown
	Check the plug unit connecting with fuel pump or short circuit or stuck leading the overheating of the fuel pump



	ER04
	Water pump     Breakdown
	Check the plug unit connecting with water pump or short circuit or stuck leading the water pump broken



	ER05
	Flame extinction
	Check if there is any bubble in the fuel pipe or not and the bubble is overlarge;   if there are too much carton in the heater burner                  


	ER06
	Breakdown of flame sensor
	Check the plug of the sensor, the positive and negative termin

 

	 ER07
	Breakdown of igniter 
	Check the plug or the igniter short circuit 

	ER08
	Breakdown of second start
	Check the fuel pipe if there is full of fuel or not, the fuel pump can work or not, any bubble in the fuel pipe, or any impurity in the fuel box jamming the fuel taking device

	ER09
	Breakdown of temperature sensor
	Check the plug, or the positive and negative terminals of the sensor leading the sensor short circuit 

	ER11
	Breakdown of Dry heating sensor
	Check the plug and if the water is cycling in the heater

	ER12
	Breakdown of Wiring harness assembly
	Check the plug if it is off, if not,please cut off the fuse


